#ifndef cbi

#define cbi(sfr, bit) (_SFR_BYTE(sfr) &= ~_BV(bit))

#endif

#ifndef sbi

#define sbi(sfr, bit) (_SFR_BYTE(sfr) |= _BV(bit))

#endif

//--------Pin definitions for the TB6612FNG Motor Driver----

#define AIN2 5  // Define motor driver pin AIN1 to digital pin 4

#define BIN2 9  // Define motor driver pin BIN1 to digital pin 6

#define AIN1 4  // Define motor driver pin AIN2 to digital pin 2

#define BIN1 8   // Define motor driver pin BIN2 to digital pin 7

#define PWMA 3  // Define motor driver PWM A to digital pin 3

#define PWMB 10

//------------------------------------------------------------

//--------Enter Line Details here---------

bool isBlackLine = 1;             //keep 1 in case of black line. In case of white line change this to 0

unsigned int lineThickness = 23;  //Enter line thickness in mm. Works best for thickness between 10 & 35

unsigned int numSensors = 5;      // Enter number of sensors as 5 or 7

//-----------------------------------------

int P, D, I, previousError, PIDvalue;

double error;

int lsp, rsp;

int lfSpeed = 80;

int currentSpeed = 45;

int sensorWeight[7] = { 4, 2, 1, 0, -1, -2, -4 };

int activeSensors;

// float Kp = 0.8;

float Kp = 0.07;

float Kd = 0.8;

float Ki = 0.0000001;

int onLine = 1;

int minValues[7], maxValues[7], threshold[7], sensorValue[7], sensorArray[7];

void setup() {

sbi(ADCSRA, ADPS2);

cbi(ADCSRA, ADPS1);

cbi(ADCSRA, ADPS0);

Serial.begin(9600);

pinMode(AIN1, OUTPUT);

pinMode(AIN2, OUTPUT);

pinMode(BIN1, OUTPUT);

pinMode(BIN2, OUTPUT);

pinMode(PWMA, OUTPUT);

pinMode(PWMB, OUTPUT);

pinMode(11, INPUT_PULLUP);  //Pushbutton

pinMode(2, INPUT_PULLUP);  //Pushbutton

pinMode(13, OUTPUT);        //LED

// pinMode(5, OUTPUT);     //standby for older carrier boards

// digitalWrite(5, HIGH);  //enables the motor driver

//in case of a newer board with motor enable jumper, you can delete these two lines which frees up digital pin 5

lineThickness = constrain(lineThickness, 10, 35);

if (numSensors == 5) {

sensorWeight[1] = 4;

sensorWeight[5] = -4;

}

}

void loop() {

// put your main code here, to run repeatedly:

while (digitalRead(2)) {}

delay(1000);

calibrate();

while (digitalRead(11)) {}

delay(1000);

while (1) {

readLine();

if (currentSpeed < lfSpeed) currentSpeed++;

if (onLine == 1) {  //PID LINE FOLLOW

linefollow();

digitalWrite(13, HIGH);

} else {

digitalWrite(13, LOW);

if (error > 0) {

motor1run(-50);

motor2run(lfSpeed);

} else {

motor1run(lfSpeed);

motor2run(-50);

}

}

}

}

void linefollow() {

error = 0;

activeSensors = 0;

if (numSensors == 7) {

for (int i = 0; i < 7; i++) {

error += sensorWeight[i] * sensorArray[i] * sensorValue[i];

activeSensors += sensorArray[i];

}

error = error / activeSensors;

}

if (numSensors == 5) {

for (int i = 1; i < 6; i++) {

error += sensorWeight[i] * sensorArray[i] * sensorValue[i];

activeSensors += sensorArray[i];

}

error = error / activeSensors;

}

P = error;

I = I + error;

D = error - previousError;

PIDvalue = (Kp * P) + (Ki * I) + (Kd * D);

previousError = error;

lsp = currentSpeed - PIDvalue;

rsp = currentSpeed + PIDvalue;

if (lsp > 255) {

lsp = 255;

}

if (lsp < 0) {

lsp = 0;

}

if (rsp > 255) {

rsp = 255;

}

if (rsp < 0) {

rsp = 0;

}

motor1run(lsp);

motor2run(rsp);

}

void calibrate() {

for (int i = 0; i < 7; i++) {

minValues[i] = analogRead(i);

maxValues[i] = analogRead(i);

}

for (int i = 0; i < 10000; i++) {

motor1run(50);

motor2run(-50);

for (int i = 0; i < 7; i++) {

if (analogRead(i) < minValues[i]) {

minValues[i] = analogRead(i);

}

if (analogRead(i) > maxValues[i]) {

maxValues[i] = analogRead(i);

}

}

}

for (int i = 0; i < 7; i++) {

threshold[i] = (minValues[i] + maxValues[i]) / 2;

Serial.print(threshold[i]);

Serial.print(" ");

}

Serial.println();

motor1run(0);

motor2run(0);

}

void readLine() {

onLine = 0;

if (numSensors == 7) {

for (int i = 0; i < 7; i++) {

if (isBlackLine) {

sensorValue[i] = map(analogRead(i), minValues[i], maxValues[i], 0, 1000);

} else {

sensorValue[i] = map(analogRead(i), minValues[i], maxValues[i], 1000, 0);

}

sensorValue[i] = constrain(sensorValue[i], 0, 1000);

sensorArray[i] = sensorValue[i] > 500;

if (sensorArray[i]) onLine = 1;

if (isBlackLine == 1 && sensorArray[i]) onLine = 1;

if (isBlackLine == 0 && !sensorValue[i]) onLine = 1;

}

}

if (numSensors == 5) {

for (int i = 1; i < 6; i++) {

if (isBlackLine) {

sensorValue[i] = map(analogRead(i), minValues[i], maxValues[i], 0, 1000);

} else {

sensorValue[i] = map(analogRead(i), minValues[i], maxValues[i], 1000, 0);

}

sensorValue[i] = constrain(sensorValue[i], 0, 1000);

sensorArray[i] = sensorValue[i] > 500;

if (sensorArray[i]) onLine = 1;

if (isBlackLine == 1 && sensorArray[i]) onLine = 1;

if (isBlackLine == 0 && !sensorValue[i]) onLine = 1;

}

}

}

//--------Function to run Motor 1-----------------

void motor2run(int motorSpeed) {

motorSpeed = constrain(motorSpeed, -255, 255);

if (motorSpeed > 0) {

digitalWrite(AIN1, 1);

digitalWrite(AIN2, 0);

analogWrite(PWMA, motorSpeed);

} else if (motorSpeed < 0) {

digitalWrite(AIN1, 0);

digitalWrite(AIN2, 1);

analogWrite(PWMA, abs(motorSpeed));

} else {

digitalWrite(AIN1, 1);

digitalWrite(AIN2, 1);

analogWrite(PWMA, 0);

}

}

//--------Function to run Motor 2-----------------

void motor1run(int motorSpeed) {

motorSpeed = constrain(motorSpeed, -255, 255);

if (motorSpeed > 0) {

digitalWrite(BIN1, 1);

digitalWrite(BIN2, 0);

analogWrite(PWMB, motorSpeed);

} else if (motorSpeed < 0) {

digitalWrite(BIN1, 0);

digitalWrite(BIN2, 1);

analogWrite(PWMB, abs(motorSpeed));

} else {

digitalWrite(BIN1, 1);

digitalWrite(BIN2, 1);

analogWrite(PWMB, 0);

}

}